WELCOME TO MODULE 2

1. 	Rigging procedure

Rigging in ALH means correlating the cockpit control travels to the actuator strokes and further to the blades angle.

Requirement of Rigging

- Change of any major component (e.g. MGB, MR blades, actuators, etc.)

- When vibrations out of limit.

- On recommendation of EO/OC.

Applicable documents

Maintenance Manual WCs 08.00.00.201 and 67.10.00.508

2.	Lower Flight Control Rigging

3.	Tools Required

(a)	Rigging Fixture ALH-SK-TP-110 \\(for adjusting the swinging arm with clinometer)

b.	 Rigging Tool ALH—9504

(c)	 Rigging Fixture ALH-9702 \\(To set Cyclic stick fore and aft and left and right positions).

(d).	ALH-SK-H-507 \\(To facilitate measurement of Main Rotor Actuator Piston travel)

f.	. ALH-SK-H-493 \\(To set Lever orientation of the Vibration Isolation System) (rigging pin)

pin

g.	 Clinometer 015-Plus-XG 45

4.	Preconditions

(a)	Helicopter leveling

- Place weighing beam(loc)

- Place jacks (loc)

- Place clinometer (loc)

-  Check for accuracy of ± 2’ in longitudinal and lateral axes in clinometer. (show)

(b)	Disconnect trim actuator (loc)

(c)	Engage the rigging pin tool ALH-SK-H-493 (loc)

(d)	Place Clinometer mounting tool ALH-SK-TP-110 (loc)

(e)	Now place clinometer and \\check for accuracy of  ±5’ (show)

(f)	In case of any deviation adjust the Fwd. Control Rod (loc)

(g)	Remove rigging pin.

5.	Procedure

Collective control channel

(a)	Turn stopper nuts of collective circuit completely backwards (loc)

(b)	Place ALH-9504 to set the pilot collective lever at lower most position. (loc)

(c)	Loosen the check nuts of (loc) control rod

(d)	Bring the input lever fully forward by adjusting tie ends (loc) \\direction

(e)	Place Vernier caliper . (loc)

(f)	Adjust the control rod till Vernier caliper reads piston movement of 0.2 to 0.4 mm (loc) 	\\direction

(g)	Tighten the check nuts (loc)

(h) 	Adjust the top stopper nut (loc) to set  collective stick at lowest position

(i)	Move collective stick (loc)  upwards till actuator input lever moves back completely 	(loc)

(j)	place Vernier caliper. (loc)

k)	move collective downwards till the Vernier caliper indicates	0.2 to 0.4 mm. (loc)

(l)	Adjust the bottom stopper nut. (loc)

(m)	Ensure for correct adjustment

(i)	move collective full fwd(loc)

(ii)	measure piston stroke by placing Vernier caliper(loc)

(iii)	 move collective full back(loc)

(iv)	measure piston stroke by placing Vernier caliper(loc).

(v)	ensure reading as 56.3 to 56.9 mm by subtracting both (text)measurements.(loc)(x - y = 56.3 to 56.9mm  )

(m)	ensure pilot collective travel as shown

(n) for co-pilot

Bring pilot stick to lowermost position(loc)

Place tool ALH-9504(loc)

Bring co-pilot stick to lowermost position , by adjusting the link rod(loc)

(o)	ensure co-pilot collective travel as shown

6 Cyclic Longitudinal Channel

(a)	Turn the stopper bolts completely backwards(loc) # longitudinal.

(b)	Attach clinometer-mounting tool ALH-9702 (loc).

(c)	place clinometer (loc)..

(d)	Move the stick forward so that the clinometer reads 10° 26’. (loc).

(e)	tighten lock nut to restricts further movement .

Loosen the check nuts of (loc) control rod

(f)	bring the input lever fully forward by adjusting control rod(loc)

(g)	place Vernier caliper(loc)

(h)	adjust tie ends (loc) till Vernier caliper reads piston movement 3.8 to 4.0 mm.(loc)

(i)	 Tighten the check nuts(loc)

(j)	Move the stick aft till the actuator input lever moves backwards completely

(k)	place Vernier caliper. (loc)

(h)	move cyclic forward till the Vernier caliper indicates 0.2 to 0.4 mm. (loc)

(l)	Engage the stopper bolt (loc)

(m)	Ensure for correct adjustment

(i)	move cyclic full fwd(loc)

(ii)	measure piston stroke by placing Vernier caliper(loc)

(iii)	 move cyclic full back(loc)

(iv)	measure piston stroke by placing Vernier caliper(loc).

(v)	ensure reading as 60.3 to 61.3 mm by subtracting both measurements.(loc)

(n)	ensure cyclic stick travel as shown

7 Cyclic lateral channel

(a)	Turn the stopper bolts (loc) completely backwards

(b)	Mount clinometer in lateral axis.(loc)

(c)	 Move pilot cyclic sick to the left so that the clinometer reads 9° 13’.

(d)	tighten lock nut to restricts further movement

(e)	bring the input lever fully forward by adjusting tie ends (loc). while stick at leftward 	stop

(f)	place Vernier caliper(loc)

(g)	adjust tie ends (loc) till Vernier caliper reads piston movement of 0.2 to 0.4 mm.(loc)

(i)	Tighten the check nuts(loc)

(j)	Move the stick rightwards till the actuator input lever moves backwards completely.

(k)	place Vernier caliper(loc)

(l)	move cyclic leftward till the Vernier caliper indicates 0.2 to 0.4 mm. (loc)

(m)	Engage the stopper bolt (loc)

(n)	Ensure for correct adjustment

(i)	move cyclic full rightward(loc)

(ii)	measure piston stroke by placing Vernier caliper(loc)

(iii)	 move cyclic full leftward(loc)

(iv)	measure piston stroke by placing Vernier caliper(loc).

(v)	ensure reading as 39.3 to 39.9 mm by subtracting both measurements.(loc)

(n)	ensure cyclic stick travel as shown

UPPER CONTROL RIGGING

1. 	Tools Required

(a)	 Calibrated Clinometer or Electrical Clinometer

(b)	Rigging tool No. ALH – 9655 for placing electrical clinometer

(c) 	Protractor, tool No ALH –SK-1304 for measurement of azimuthal position of blade 	No.1

(d)	 Pointer tool No. ALH – 9624

(e)	 Tool No. ALH – 9646 for locking the cyclic stick during collective angles 	measurement

Pre-conditions

- place clinometer (loc)

check for accuracy of ± 2’ in both longitudinal and lateral axes in clinometer(check)

- Rigging of lower control system to be completed(text)

-All measurements have to be perpendicular to the blade axis(text)

-place rigging tool  No. ALH – 9655 on blade number 1(yellow ) (loc)

-place clinometer on tool(loc)

-place tool No ALH –SK-1304  on MGB(loc)

-insert Pointer tool No. ALH – 9624 in main bolt to measure angle(loc)

9 Collective input Vs. Blade Angle

1.	Pre condition

Use Tool No. ALH – 9646 for locking the cyclic (loc)

Use protractor to bring yellow blade at 0 degree(loc)

2.	Pull Collective lever Max. Up

- measure reading on clinometer (loc) and enter in table

-now take measurement of yellow blade at 90, 180, 270 degree

-take the mean value and enter in table

-ensure reading as 9 degree+/-10 min

3.	Collective lever Max. Down

- again Use protractor to bring yellow blade at 0 degree(loc)

-measure the reading and enter in table

-now take measurement of yellow blade at 90, 180, 270 degree

-take the mean value and enter in table

-ensure reading as -8 degree 50 min to -9 degree

4.	If the mean value not met, adjust the length of the connecting rod by turning . (For 1 deg , give 1.14 turns)(text)

Cyclic input Vs. Blade Angle

(a)	put collective down

(b) 	measure the blade angle on clinomter as per condition shown in table

| s.no | Blade posn | Condition | reading |
| --- | --- | --- | --- |

(c)	If the value not met, adjust the length of the connecting rod by turning.

Collective, Longitudinal and Lateral Combined

Measure the blade angle as per conditions shown

| Blade posn | Condition | Reading |
| --- | --- | --- |

Final Settings

- Wire lock all nuts

- Recharge hyd package

- Check all connections

- Check for any FOD

- Deliver aircraft for next vibration checks.